IBM 7090/7094 Simulator Usage

01-Dec-2008

COPYRIGHT NOTICE

The following copyright notice applies to the SIMH source, binary, and documentation:

Original code published in 1993-2008, written by Robert M Supnik

Copyright (c) 1993-2008, Robert M Supnik

Permission is hereby granted, free of charge, to any person obtaining a copy of this software and associated documentation files (the "Software"), to deal in the Software without restriction, including without limitation the rights to use, copy, modify, merge, publish, distribute, sublicense, and/or sell copies of the Software, and to permit persons to whom the Software is furnished to do so, subject to the following conditions:

The above copyright notice and this permission notice shall be included in all copies or substantial portions of the Software.

THE SOFTWARE IS PROVIDED "AS IS", WITHOUT WARRANTY OF ANY KIND, EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE AND NONINFRINGEMENT.  IN NO EVENT SHALL ROBERT M SUPNIK BE LIABLE FOR ANY CLAIM, DAMAGES OR OTHER LIABILITY, WHETHER IN AN ACTION OF CONTRACT, TORT OR OTHERWISE, ARISING FROM, OUT OF OR IN

CONNECTION WITH THE SOFTWARE OR THE USE OR OTHER DEALINGS IN THE  SOFTWARE.

Except as contained in this notice, the name of Robert M Supnik shall not be used in advertising or otherwise to promote the sale, use or other dealings in this Software without prior written authorization from Robert M Supnik.

1	Simulator Files	3

2	IBM 7090/7094 Features	3

2.1	CPU	4

2.2	Interval Timer (CLK)	5

2.3	I/O Channels (CHANA..CHANH)	5

2.4	Channel A Devices	6

2.4.1	711 Card Reader (CDR)	6

2.4.2	721 Card Punch (CDP)	7

2.4.3	716 Line Printer (LPT)	8

2.5	729 Magnetic Tape (MTA…MTH)	9

2.6	7631 File Control (DSK)	10

2.7	7289 High-Speed Drum (DRM)	11

2.8	7750 Communications Controller (COM and COML)	11

3	Symbolic Display and Input	13

4	Character Sets	13

This memorandum documents the IBM 7094 simulator.

# Simulator Files

To compile the IBM 7094, you must define USE_INT64 as part of the compilation

command line.

sim/		scp.h

sim_console.h

sim_defs.h

sim_fio.h

sim_rev.h

sim_sock.h

sim_tape.h

sim_timer.h

sim_tmxr.h

scp.c

sim_console.c

sim_fio.c

sim_sock.c

sim_tape.c

sim_timer.c

sim_tmxr.c

sim/i7094/	i7094_defs.h

i7094_dat.h

i7094_cd.c

i7094_clk.c

i7094_com.c

i7094_cpu.c

i7094_cpu1.c

i7094_drm.c

i7094_dsk.c

i7094_io.c

i7094_lp.c

i7094_mt.c

i7094_sys.c

# IBM 7090/7094 Features

The IBM 7090/7094 simulator is configured as follows:

device name(s)	simulates

CPU			7090, 7094, or 7094 with CTSS RPQ's CPU with 32KW, 32KW,

or 64KW of memory, respectively

CLK			interval timer (RPQ F89349) and Chronolog clock

CHANA			7607 channel (required)

CHANB..CHANH	additional 7607, 7289, or 7909 channels

MTA			magnetic tape controller, channel A (required)

MTB..MTH		additional magnetic tape controllers, channels B to H

CDR			711 card reader

CDP			721 card punch

LPT			716 line printer

DSK			7631 file control with up to 10 modules (disks or drums)

DRM			7289 fast drum control

COM			7750 communications control

COML			7750 communications lines

Channels B through H, the corresponding magnetic tape controllers, and the file control, drum control, and communications control are initially set DISABLED.  The file control, drum control, and communications control can be assigned to any channel in the range B through H.

The 7090/7094 simulator implements several unique stop condition:

- Undefined CPU instruction

- Undefined channel instruction

- XEC nesting exceeds limit

- Divide check on a divide and halt instruction

- Select of a non-existent channel

- 7607 select of a 7909 channel

- Write select of a write protected device

- Invalid file control format

- Invalid message to 7750

- No buffer storage available for input character on 7750

- No buffer storage available for output character on 7750

The LOAD command is not implemented.

## CPU

The CPU options allow the user to specify a 7090, a 7094, or a 7094 with CTSS RPQ's.

SET CPU 7090			7090

SET CPU 7094			Standard 7094

SET CPU CTSS			7094 with CTSS RPQ's.

Memory size is 32KW on a 7090 or 7094 CPU, 64KW on a CTSS CPU.  CTSS mode enables access to the Chronolog clock as magtape unit A7.

CPU registers include the visible state of the processor as well as the control registers for the interrupt system.

name			size		comments

PC			15		program counter

AC			38		accumulator

MQ			36		multiplier-quotient

SI			36		storage indicators

KEYS			36		front panel keys

XR1..XR7		15		index registers 1 to 7

[7090 uses only XR1, XR2, XR4]

SS1..SS6		1		sense switches 1 to 6

SL1..4		1		sense lights 1 to 4

OVF			1		AC overflow indicator

MQO			1		MQ overflow indicator

DVC			1		divide check indicator

IOC			1		I/O check indicator

TTRAP			1		transfer trap enable

CTRAP			1		copy trap enable

STRAP			1		select trap enable

FTRAP			1		floating point trap enable

STORN			1		storage nullification mode enable

MULTI			1		multiple-tag mode enable

[always 1 on 7090]

CHREQ			8		channel request flags

CHTR_PEND		1		channel trap pending

CHTR_INHT		1		channel trap inhibit

CHTR_INHI		1		channel trap instruction inhibit

CHTR_ENAB		30		channel trap enable flags

USERM			1		user mode flag [CTSS only]

IMEM			1		instruction B-core flag [CTSS only]

DMEM			1		data B-core flag [CTSS only]

RELOC			8		relocation base block [CTSS only]

START			8		address start block [CTSS only]

LIMIT			8		address limit block [CTSS only]

OLDPC			15		PC at start of instruction

PCQ[0:63]		15		PC prior to last jump or interrupt;

most recent PC change first

HTPEND		1		halt and transfer pending

HTADDR		15		halt and transfer address

XECMAX		8		XEC chain limit

WRU			8		interrupt character

STOP_ILL		1		stop on undefined instruction

The CPU can maintain a history of the most recently executed instructions.  This is controlled by the SET CPU HISTORY and SHOW CPU HISTORY commands:

SET CPU HISTORY			clear history buffer

SET CPU HISTORY=0			disable history

SET CPU HISTORY=n			enable history, length = n

SHOW CPU HISTORY			print CPU history

SHOW CPU HISTORY=n		print first n entries of CPU history

If switch -C is set, channel commands are also included in the CPU history.  The maximum length for the history is 262144 entries.

## Interval Timer (CLK)

The timer (CLK) implements a 60Hz interval timer.  It is disabled by default.

The timer has the following registers:

name			size		comments

TRAP			1		interval timer trap flag

TIME			24		tick delay

## I/O Channels (CHANA..CHANH)

The 709X supports up to 8 channels.  Channel models include

7607			standard multiplexer channel

7289			high speed drum channel

7909			advanced capabilities channel

Channel A is required and is always a 7607.  Channels B through H are disabled by default.  Channels B through H can be enabled.  When a channel is enabled, the attached device must also be specified:

SET CHANx ENABLED{=[729|TAPE|7289|DRUM|7631|FILE|7750|COMM]}

If no device type is specified, TAPE is assumed.

Setting a channel to 729 (synonym TAPE) defines the channel as a 7607 and enables the corresponding magnetic tape controller.  Setting a channel to 7289 (synonym DRUM) defines the channel as a 7289 and enables the high-speed drum control.  Setting a channel to 7631 defines the channel as a 7909 and enables the file control.  Setting a channel to 7750 (synonym COMM) defines the channel as a 7909 and enables the communications control.  Only one high-speed drum control, one file control, and one communications control are supported per system.

As an example, the following commands set up the 1971 CTSS configuration:

SET CPU CTSS

SET CLK ENABLED

SET CHANB ENABLED=TAPE

SET CHANC ENABLED=DISK

SET CHANE ENABLED=COMM

SET CHANG ENABLED=DRUM

Channels have the following registers:

name		type		size		comments

STATE		all		8		channel state

DSC		7607,7289	4		data select

DSU		7607,7289	9		data select unit

NDSC		7607,7289	4		non-data select

NDSU		7607,7289	9		non-data select unit

FLAGS		all		30		channel flags

IDF		all		2		input data flags

OP		all		5		channel opcode

CLC		all		16		channel location counter

WC		all		15		channel word counter

CA		all		16		channel current address

AR		all		36		channel assembly register

CND		7909		6		channel interrupt conditions

LCC		7909		6		channel control counter

SMS		7909		7		channel options mask

## Channel A Devices

### 711 Card Reader (CDR)

The card reader (CDR) reads data from a disk file.  Cards are simulated as ASCII lines with terminating newlines.  The POS register specifies the number of the next data item to be read.  Thus, by changing POS, the user can backspace or advance the reader.

Card reader files can either be text (one character per column) or column binary (two characters per column).  The file type can be specified with a set command:

SET CDR TEXT			set text mode

SET CDR BINARY			set column binary mode

or in the ATTACH command:

ATT -T CDR <file>			set text mode

ATT CDR <file>.TXT		set text mode

ATT -C CDR <file>			set column binary mode

ATT CDR <file>.CBN		set column binary mode

The card reader supports the BOOT command.  BOOT CDR starts the standard card reader bootstrap at location 0.

The card reader implements these registers:

name			size		comments

STATE			2		reader state

BPTR			5		binary buffer pointer

BUF[0:23]		36		binary buffer

POS			32		position in the input file

TSTART		24		card start delay

TSTOP			24		card stop delay

TLEFT			24		delay between row halves

TRIGHT		24		delay between rows

Error handling is as follows:

error			processed as

not attached	report error and stop

end of file		out of cards

OS I/O error	report error and stop

### 721 Card Punch (CDP)

The card reader (CDP) writes data to a disk file.  Cards are simulated as ASCII lines with terminating newlines.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the punch.

Card punch files can either be text (one character per column) or column binary (two characters per column).  The file type can be specified with a set command:

SET CDP TEXT			set text mode

SET CDP BINARY			set column binary mode

or in the ATTACH command:

ATT -T CDP <file>			set text mode

ATT CDP <file>.TXT		set text mode

ATT -C CDR <file>			set column binary mode

ATT CDP <file>.CBN		set column binary mode

The card punch supports both the business (1403 print chain A) and Fortran (1403 print chain H) character sets:

SET CDP BUSINESS			business character set

SET CDP FORTRAN			Fortran character set

The Fortran character set is the default.

The card punch implements these registers:

name			size		comments

STATE			2		reader state

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

BPTR			5		binary buffer pointer

BUF[0:23]		36		binary buffer

POS			32		position in the output file

TSTART		24		card start delay

TSTOP			24		card stop delay

TLEFT			24		delay between row halves

TRIGHT		24		delay between rows

Error handling is as follows:

error			processed as

not attached	report error and stop

OS I/O error	report error and stop

### 716 Line Printer (LPT)

The line printer (LPT) writes data to a disk file as ASCII text with terminating newlines.  The POS register specifies the number of the next data item to be written.  Thus, by changing POS, the user can backspace or advance the printer.

The line printer implements both 48- and 64-character print chains:

SET LPT 64				64-character print chain

SET LPT 48				48-character print chain

The line printer also implements both the business (1403 print chain A) and Fortran (1403 H chain) character sets:

SET LPT BUSINESS			business print character set

SET LPT FORTRAN			Fortran character set

The default is 64 characters, Fortran set.

Finally, because the line printer was used for status output messages, its output can be redirected to the controlling terminal window if no file is attached:

SET LPT DEFAULT			default output to console window

SET LPT NODEFAULT			no default output, error if not attached

The line printer implements these registers:

name			size		comments

STATE			2		printer state

CMD			2		printer command

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

BPTR			5		binary buffer pointer

BUF[0:23]		36		binary buffer

EBUF[0:22]		36		echo buffer

POS			32		position in the output file

TSTART		24		line start delay

TSTOP			24		line print delay

TLEFT			24		delay between row halves

TRIGHT		24		delay between rows

Error handling is as follows:

error			processed as

not attached	report error and stop

OS I/O error	report error and stop

## 729 Magnetic Tape (MTA…MTH)

Every 7607 channel can support up to ten seven-track magnetic tape units (MTx1…MTx10).  Magnetic tape options include the ability to make units write enabled or write locked.

SET MTn LOCKED			set unit n write locked

SET MTn WRITEENABLED		set unit n write enabled

Magnetic tape units can be set to a specific reel capacity in MB, or to unlimited capacity:

SET MTn CAPAC=m			set unit n capacity to m MB (0 = unlimited)

SHOW MTn CAPAC			show unit n capacity in MB

Units can also be set ENABLED or DISABLED.  The magnetic tape simulator supports the BOOT command.  BOOT MTxn starts the standard magnetic tape load program at location 0.

The magnetic tape controllers implement the following registers:

name			size		comments

UNIT			5		unit select code

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

BPTR			16		buffer pointer

BLNT			16		buffer length

BUF			7		character buffer (with parity)

TWEF			24		wait time for end of file

TSHORT		24		wait time for “immediate” commands

TSTART		24		wait time for unit start

TSTOP			24		wait time for unit stop

TWORD			24		wait time between word transfers

UST[1:10]		5		unit state, drives 1 to 10

POS[1:10]		32		position, drives 1 to 10

Error handling is as follows:

error			processed as

not attached	report error and stop

end of file		set error indicator

OS I/O error	print error message

set error indicator

report error and stop

## 7631 File Control (DSK)

The 7631 file control supports up to ten devices, which can be 7320 drums, 1301 disks, 1302 disks, or 2302 disks.  Unit types are specified with the SET command.  The type can be set only if the unit (and the next unit in sequence) is unattached, and the unit number is even:

SET DSKn 7320			unit n is a drum (unit n+1 is disabled)

SET DSKn 1301			unit n is a 1301 disk (unit n+1 is the same)

SET DSKn 1302			unit n is a 1302 disk (unit n+1 is the same)

SET DSKn 2302			unit n is a 2302 disk (unit n+1 is the same)

Units can be SET ENABLED or DISABLED.  In addition, units can be set to enable or disable formatting:

SET DSKn FORMAT			enable formatting

SET DSKn NOFORMAT			disable formatting

Formatting is disabled by default.  The current format can be shown with the command:

SHOW DSKn FORMAT			display format information

The 7631 implements the following registers:

name			size		comments

STATE			6		file control state

ACCESS		1		currently selected access

MODULE		4		currently selected module (0-9)

RECORD		36		record address (6 BCD characters)

MODE			4		disk I/O mode

SENSE			60		sense data (10 BCD characters)

BCDCMD		60		most recent command (10 BCD characters)

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

STOP			1		channel stop flag

FCNTR			13		format track character counter

BUF[0:999]		36		track buffer

RBASE			10		offset to record base

RPTR			10		offset to current word

RLIM			10		offset to record end

STIME			24		seek delay

RTIME			24		rotational delay

WTIME			24		inter-word delay

GTIME			24		end-of-sector (gap) delay

CTIME			24		command processing delay

TRACK[0:19]		10		current track number

[0:9] module n, access 0

[10:19] module n, access 1

Error handling is as follows:

error			processed as

not attached	report error and stop

OS I/O error	report error and stop

## 7289 High-Speed Drum (DRM)

The 7289 (also known as the 7320A) high-speed drum was a late addition to CTSS.  Very little is known about the device, other than what is used in the CTSS sources.

The drum implements these registers:

name			size		comments

STATE			2		drum state

DA			18		drum address register

OP			1		read/write flag

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

TIME			24		inter-word delay

Error handling is as follows:

error	 		processed as

not attached	report error and stop

Drum data files are buffered in memory; therefore, end of file and OS I/O errors cannot occur.

## 7750 Communications Controller (COM and COML)

The 7750 is modeled as a terminal multiplexer with 33 lines.  It consists of two devices: COM is the multiplexer controller, and COML is the individual lines. For the first 32 lines, the 7750 performs input and output through Telnet sessions connected via a user-specified listening port; the 33rd line is permanently attached to the simulator console window.  The ATTACH command specifies the port to be used for Telnet sessions:

ATTACH COM <port>			set up listening port

where port is a decimal number between 1 and 65535 that is not being used for other TCP/IP activities.

Each line (each unit of COML) can be set to one of two modes: KSR-35 and KSR-37.  In KSR-35 mode, lower case input and output characters are converted automatically to upper case, and parity is ignored.  In KSR-37 mode, lower case characters are left alone, and even parity is generated on input.  KSR-37 is the default.

Once COM is attached and the simulator is running, the 7750 listens for connections on the specified port.  It assumes that any incoming connection is a Telnet connection.  A connection remains open until disconnected either by the Telnet client, a SET COM DISCONNECT command, or a DETACH COM command.

The 7750 implements the following special SHOW commands

SHOW COM CONNECTIONS		displays current connections to the 7750

SHOW COM STATISTICS		displays statistics for active connections

SHOW COM FREEQ			displays the character buffer free list

SHOW COM INPQ			displays the character input queue

SHOW COM OUTQ			displays the output queues for all lines

SHOW COMn OUTQ			displays the output queue for line n

The 7750 implements the following special SET commands:

SET COM DISCONNECT=n		disconnect line n

SET COMLn DISCONNECT		disconnect line n

SET COMLn LOG=filename		log output of line n to filename

SET COMLn NOLOG			disable logging and close log file

SET COMLn KSR35			set line n to KSR-35

SET COMLn KSR37			set line n to KSR-37

The controller (COM) implements these registers:

name			size		comments

ENABLE		1		enable flag

STATE			6		controller state

MSGNUM		12		input message sequence number

CHOB			36		channel output buffer

CHOBV			1		output buffer valid flag

STOP			1		channel stop flag

BUF[0:119]		36		channel buffer

BPTR			7		channel buffer pointer

BLIM			7		channel buffer limit

FREEQ[0:1]		16		free queue header

INPQ[0:1]		16		input queue header

OUTQ[0:65]		16		output queue headers, lines 0 to 32

PKTB[0:32767]	16		character buffer entries

Queue headers consist of two 16b words; both are subscripts into the character buffer array.  The first word is the buffer subscript for the queue head; the second is the buffer subscript for the queue tail.  In an empty queue, both words are 0.

Character buffer entries also consist of two 16b words.  The first is the buffer subscript for the next entry in the queue; 0 indicates end of queue.  The second is the data element, typically a 12b character.

The lines (COML) implements these registers:

name			size		comments

TIME[0:32]		24		transmit time, lines 0 to 32

The 7750 does not support save and restore.  All open connections, except the permanent connection to the console window, are lost when the simulator shuts down or COM is detached.

# Symbolic Display and Input

The IBM 7094 simulator implements symbolic display and input.  Display is controlled by command line switches:

-c			display as character

-s			display as character string

-m			display instruction mnemonics

-i			display as 7607 IO instruction

-n			display as 7909 IO instruction

Character and string display is further qualified by switches that specify the character coding and conversion conventions:

-b			BCD data (default is nine-code)

-a			business character set (default is Fortran)

The default data coding is nine-code, and the default character set is Fortran.  Note that 7094 BCD and IBM 1401 BCD differ in one important regard: the 7094 interprets 0 as code 20, the 1401 as code 12.

Input parsing is controlled by the first character typed in or by command line switches:

' or -c		character

" or -s		string

alphabetic		instruction mnemonic

numeric		octal number

Instruction input uses standard 7094 assembler syntax.  There are two basic instruction classes: memory reference and index reference.

Memory reference instructions have the format

memref{*} address{,tag}

Index reference instructions have the format

idxref{*} address,{tag},decrement

Specific instructions may disallow indirect addressing or limit the size of the tag, address, or decrement fields.

Channel (I/O) instructions have the same basic two formats.

# Character Sets

The IBM 7094  uses a 6b character code called 9-code, a variation (with permuted zones) of the ubiquitous BCD (binary coded decimal).  The 7094 also uses BCD for communicating with the card reader/punch and the line printer.  In both 9-code and BCD, some of the characters have no equivalent in ASCII and require different representations:

9-	    ASCII		IBM 7094		print

code	representation	character		chains

00	0

01	1

02	2

03	3

04	4

05	5

06	6

07	7

10	8

11	9

12	^

13	# or =					# in A, = in H

14	@ or '					@ in A, ' in H

15	:						blank in A, H 48 char

16	>						blank in A, H 48 char

17	{			tape mark		blank in A, H 48 char

20	& or +					& in A, + in H

21	A

22	B

23	C

24	D

25	E

26	F

27	G

30	H

31	I

32	?

33	.

34	)

35	[						blank in A, H 48 char

36	<						blank in A, H 48 char

37	}			group mark		blank in A, H 48 char

40	-

41	J

42	K

43	L

44	M

45	N

46	O

47	P

50	Q

51	R

52	!

53	$

54	*

55	]						blank in A, H 48 char

56	;						blank in A, H 48 char

57	_			delta			blank in A, H 48 char

60	space

61	/

62	S

63	T

64	U

65	V

66	W

67	X

70	Y

71	Z

72	|			record mark

73	,

74	% or (					% in A, ( in H

75	~						blank in A, H 48 char

76	\						blank in A, H 48 char

77	"						blank in A, H 48 char

BCD	    ASCII		IBM 7094		print

code	representation	character		chains

00	space

01	1

02	2

03	3

04	4

05	5

06	6

07	7

10	8

11	9

12	0

13	# or =					# in A, = in H

14	@ or '					@ in A, ' in H

15	:						blank in A, H 48 char

16	>						blank in A, H 48 char

17	{			tape mark		blank in A, H 48 char

20	^						blank in A, H 48 char

21	/

22	S

23	T

24	U

25	V

26	W

27	X

30	Y

31	Z

32	|			record mark

33	,

34	% or (					% in A, ( in H

35	~						blank in A, H 48 char

36	\						blank in A, H 48 char

37	"						blank in A, H 48 char

40	-

41	J

42	K

43	L

44	M

45	N

46	O

47	P

50	Q

51	R

52	!

53	$

54	*

55	]						blank in A, H 48 char

56	;			delta			blank in A, H 48 char

57	_						blank in A, H 48 char

60	&

61	A

62	B

63	C

64	D

65	E

66	F

67	G

70	H

71	I

72	?

73	.

74	)

75	[						blank in A, H 48 char

76	<						blank in A, H 48 char

77	}			group mark		blank in A, H 48 char